	Symbol
	Terminal name
	Symbol
	Terminal Name

	E01
	POWER GROUND
	G-
	DISTRIBUTOR

	E02
	POWER GROUND
	T
	CHECK CONNECTOR

	No. 1
	INJECTOR
	OX1
	CHECK CONNECTOR

	No. 2
	INJECTOR
	OX2
	CHECK CONNECTOR

	No. 3
	INJECTOR
	KNK
	KNOCK CONTROL SENSOR

	No. 4
	INJECTOR
	PIM
	TURBO CHARGING PRESSURE SENSOR

	STJ
	COLD START INJECTOR
	THW
	WATER TEMP SENSOR

	EGR
	EGR VSV
	IDL
	THROTTLE POSITION SENSOR

	RSC
	ISC VALVE
	THA1
	AIR TEMP SENSOR

	RSO
	ISC VALVE
	VTA
	THROTTLE POSITION SENSOR

	HT
	OXYGEN SENSOR HEATER
	VS
	AIR FLOW METER

	IGT
	IGNITER
	VC
	SENSOR POWER SOURCE

	TPC1
	TURBO CHARGING PRESSURE VSV
	E2
	SENSOR GROUND

	TVIS
	INTAKE AIR VSV
	STA
	STARTER SWITCH

	FC
	CIRCUIT OPENING RELAY
	AC1
	A/C MAGNET SWITCH

	IGF
	IGNITER
	ACT
	A/C AMPLIFIER

	G2
	DISTRIBUTOR
	SPD
	SPEED SENSOR

	G1
	DISTRIBUTOR
	FPR
	FUEL PUMP RELAY

	NE
	DISTRIBUTOR
	W
	WARNING LIGHT

	E1
	ENGINE GROUND
	STP
	STOP LIGHT SWITCH

	+B1
	MAIN RELAY
	ELS
	TAILLIGHT AND DEFOGGER

	BATT
	BATTERY
	+B
	MAIN RELAY

	VF
	CHECK CONNECTOR
	
	

	
	
	
	

	
	
	
	


Left Connector

	Top
	E01
	No. 1
	No. 2
	RSO
	RSC
	HT
	STJ
	EGR
	G2
	NE
	IGF
	TPC
	TVIS

	Bottom
	E02
	No. 3
	No. 4
	EVP
	
	NEO
	IGT
	
	G1
	G-
	
	RLY
	E1


Middle Connector

	Top
	VF
	VTA2
	OX
	PIM
	THW
	THA
	VS
	VC

	Bottom
	IDL2
	TE1
	TE2
	KNK
	IDL1
	VTA1
	THG
	E2


Right Connector

	Top
	STA
	AC
	SPD
	PS
	PSCT
	FPR
	W
	STP
	EGW
	ELS
	BATT

	Bottom
	ECO
	ACT
	TR1
	TR2
	VTO1
	VTO2
	CC0
	ABS
	FC
	+B1
	+B


	Top
	ID
	E01
	No. 1
	No. 2
	RSO
	RSC
	HT
	STJ
	EGR
	G2
	NE
	IGF
	TPC
	TVIS

	
	Main Colour
	White
	Red
	Green
	Green
	Green
	Pink
	Green
	Green
	Blue
	Yellow
	Black
	Blue
	Pale Green

	
	Stripe Colour
	Black
	
	
	Black
	White
	Black
	Red
	Red
	
	
	Yellow
	Red
	

	
	Band Colour
	
	
	
	
	
	
	
	
	
	
	Red
	
	

	Bottom
	ID
	E02
	No. 3
	No. 4
	EVP
	
	NEO
	IGT
	
	G1
	G-
	
	RLY
	E1

	
	Main Colour
	White
	Yellow
	Blue
	Pink
	NO WIRE
	NO WIRE
	White
	NO WIRE
	Red
	Black
	NO WIRE
	Blue
	Brown

	
	Stripe Colour
	Black
	
	
	
	
	
	
	
	
	
	
	Yellow
	

	
	Band Colour
	
	
	
	Red
	
	
	Red
	
	
	
	
	Red
	


	Top
	ID
	VF
	VTA2
	OX
	PIM
	THW
	THA
	VS
	VC

	
	Main Colour
	Purple
	NO WIRE
	White
	Red
	Red
	Grey
	Grey
	Pink

	
	Stripe Colour
	Yellow
	
	
	White
	
	
	Black
	Blue

	
	Band Colour
	Red
	
	
	Blue
	Blue
	Red
	Red
	Red

	Bottom
	ID
	IDL2
	TE1
	TE2
	KNK
	IDL1
	VTA1
	THG
	E2

	
	Main Colour
	NO WIRE
	Orange
	Pale Green
	Translucent
	Pink
	Pink
	NO WIRE
	Brown

	
	Stripe Colour
	
	
	
	
	
	Black
	
	

	
	Band Colour
	
	Red
	Red
	
	
	
	
	Blue


	Top
	ID
	STA
	AC
	SPD
	PS
	PSCT
	FPR
	W
	STP
	EGW
	ELS
	BATT

	
	Main Colour
	Black
	Black
	Blue
	NO WIRE
	Yellow
	Green
	Pink
	Green
	NO WIRE
	Orange
	Pink

	
	Stripe Colour
	
	White
	White
	
	
	Red
	
	White
	
	
	

	
	Band Colour
	Silver
	Silver
	Silver
	
	Silver
	Silver
	Silver
	Silver
	
	Silver
	Silver

	Bottom
	ID
	ECO
	ACT
	TR1
	TR2
	VTO1
	VTO2
	CCO
	ABS
	FC
	+B1
	+B

	
	Main Colour
	NO WIRE
	Pale Green
	NO WIRE
	NO WIRE
	NO WIRE
	NO WIRE
	NO WIRE
	Pink
	Green
	Black
	Black

	
	Stripe Colour
	
	Black
	
	
	
	
	
	Black
	
	Yellow
	Yellow

	
	Band Colour
	
	Silver
	
	
	
	
	
	Silver
	Silver
	Silver
	


