ok this is for anyone who needs to hard wire a greddy turbo timer into a st185 

-connect green wire to orange+black 

-connect red wire to red+black 

-leave out blue wire altogether. 

i have it wired like this and it works fine. 

thanks to all the ppl who helped me with mine.

hey i just installed a GReddy auto timer on a 90 st185 

n when i wired the speed sensor the auto timer is reading double the value 

like... say im driving at 20km/h it reads 40 n etc 

any problems like this b4? 

did i wire up the right wire? 

i think it was the top row n 3rd from the left pin 

from the 22 pin ecu harness 

thanks in advance

You have the wire right, but you need to adjust the speed on the turbo timer itself. There are instructions in the manual on how to do it. You have to be driving at 40 km/hr to do it.

This is how the Greddy TT hooks up to the TT harness.
[image: image1.png]|
W/





BTW, that is when looking at the wire side of the small connector. 

You can make the insatllation much easier using the TT harness. 

It costs about $16. Part number for ST185 is TT14-13
It's the picture of the optional harness. But you can splice into the stock harness the same way the optional harness is spliced into. The stock connectors are under the iginiton key. You can access to them by removing the steering under cover panels (one plastic and one metal) When removing the plastic one, be sure to remove 2 bolts below the ignition key. They are hiden behind a small black plastic piece right under the ignition key. Use small flat head screw driver to pry it out. Also you have to remove the door sill first. 

BTW, your local speed shop might carry this TT harness. It is the same as MR2 91-95.

